# Lecture 6 – Bionic Hearts

PART 1 – Cardiovascular physiology (not examinable)

- Purpose

  - Transports oxygen to body cells and takes away carbon dioxide (mainly)

  - Regulates body temperature and water content of cells

  - Carries white blood cells and antibodies to protect against disease

- Circulation

  - Left Heart has oxygen-rich blood

    - Pumps blood into aorta (largest vessel) out of left ventricle

    - Blood → Collarbones → limbs + legs

    - Blood → aortic arch → brain and body (eg. Kidney and guts)

  - When blood moves, blood vessels get smaller

    - Largest = aorta

    - Smallest = tissue capillaries

    - Oxygen leaves blood for tissues, while CO2 leaves cells into blood

  - Blood in vena cava → Right heart

    - No oxygen in right heart, since all gone to cells

    - REPEAT: Pumps deoxygenated blood to lungs to pick up oxygen and drop CO2, and returns to left heart

  - Ventricles = main driving force

    - Left Ventricle bigger than right ventricle

      - 5-10x more energy, ∴ more pressure

      - More skin on outside, so muscles squish more blood out

  - Atria/atrium

    - Collects blood before it goes into ventricles

    - Right atrium receives deoxygenated blood from veins

    - Left atrium oxygenates blood from pulmonary veins

  - Heart has 4 valves

    - Blood is one-directional

    - Heart is pulsatile (relaxes and contracts), to continue flowing blood

  - Question: What happens to pressure and flow in systemic circulation when the left ventricle fails? (see pic)

    - Flow and pressure goes down

    - Less force, less energy

    - Min flow rate/pressure to allow blood to into tissue

  - Question: What happens to pressure and flow in pulmonary circulation when the left ventricle fails? (see pic)

    - Pressure rises as right heart pumps strongly

    - Accumulation of blood, since left heart does not function

  - Question: Do you think that the body compensates for LV heart failure? Yes/No?

    - Yes

    - Needs to maintain cardiac output (amount of blood pumped/min) and RAP (blood pressure of right atrium)

    - Heart beats faster to maintain → attempts to gain oxygen in heart → increase muscle mass (due to overwork) → heart enlargement

- Heart Disease

  - Causes: dysfunctional heart valves, virus, birth defects or coronary artery disease (damage in heart’s blood vessel)

  - Heart has to be alive when taken out (eg. On life-support) ∴ Not-enough donors

PART 2 – Mechanical Therapy (Examinable)

- Ventricular Assist Devices (VADs) supports failing ventricles

  - 1st Gen

    - Tried to mimic heart

    - Pneumatic (air or pressurized gas)/electric

    - Sits outside of body (usually)

      - Can see blood going in and out

      - Connected to large air compressor ∴ lack mobility → poor quality of life

      - Very uncomfortable

    - Prone to infection, especially around tube gap into body

    - Potential to create blood clots in valves

    - Provide short term support

    - Aged out in developed countries because of poor functionality

  - 2nd Gen

    - Adapted a more continuous air flow; centrifugal or axial

    - Longer support (approx. 8 years) compared to 12-24 months

    - Steady decrease of survival over prolonged time

    - Small – sized → more mobility

    - Electronically driven

      - No pump = no air compressors

      - Less power requirements

    - Disadvantages:

      - Mechanical contact → formation of blood clots

        - Goes through impeller → breaks it → loses functionality

        - Size Growth → jams impeller → VAD breaks down

        - Goes through untouched → travels to patient’s brain → death

      - Blood Damage

      - Infection

    - DESIGN:

      - Impeller = spiral disk

        - sucks blood and pushes it back out with higher flow rate

        - Needs bearing to spin

      - Thrust Bearing

        - Stop impeller from moving

      - Point of contact with bearings

        - Very high shear stress

        - Blood cells caught and torn to shreds

  - 3rd Gen

    - Continuous flow: axial + centrifugal

    - Completely implantable, with small cable exiting body

    - No mechanical wear → impeller has no contact with any parts

    - DESIGN:

      - Hydrodynamic Bearings

        - Disk spins fast → little slope on bottom generates lift force → allows impeller disk to float above

        - High-pressure fluid (blood) underneath rotor

        - Rotor surfs on top of blood

        - Clearance (h = height between rotor/impeller and housing) needs to be large enough to prevent blood trauma

        - Small Clearance (h) required to generate pressure

          - Too big, won’t generate lift force

          - Blood is thick, so high shear rates can damage it

∴ Clearance needs to be big enough to allow blood undamaged, but small enough to generate enough lift force to lift impeller off housing

        - Changes in clearance requires change in speed or surface area

        - Min start-up speed required to maintain lift force

          - Not fast enough = drags impeller along casing → damage blood cells

      - Electromagnetic bearings

        - Magnets on either side of the impeller suspends the impeller between them

        - Can do large gaps → prevents blood cell damage

        - Magnets are large and bulky

        - More power than hydrodynamic

        - Requires additional sensors for control over rotor position

- General Complications wit VADs

- Bleeding is biggest

  - Patients often take anti-clotting meds

  - Greater risk of death if they get a cut/graze

- Infection

- Potentially due to trauma at exit site

  - Patient’s movement causes driveline (cable) to wiggle → damage suture, etc.

  - Easier for bacteria to enter

- Require daily monitoring and dressing changes

- Very difficult to treat

- Driveline design and coating can cause VAD infection

- Have tried transcutaneous energy transfer (wireless)

- Right Heart failure (see right)

  - Right ventricle needs to keep up with left side

  - LVAD support, more common

  - Can use two 3rd gen LVAD’s, since Bivads have poorer survival rates

  - Cause respiratory failure

- Neurological dysfunction

  - Due to thrombus (clots) forming in ventricle or LVAD

  - Important to optimize LVAD and ventricular flow dynamics

  - Cannula design reduces neurologic complications from ~23% → ~4%

- Cardiac Arrhythmia

  - Irregular heartbeat

  - Cause reduced blood flow, right heart failure, thrombus

  - Caused by cannula-heart interference

  - Heart can be sucked onto cannula by negative pressure created by LVAD

- Total Artificial Hearts (TAH)

  - Completely replaces the heart, ie. takes out heart and replaces completely

  - Used if:

    - Native heart cannot support life

    - Ineligible for transplant (ie. Not enough heart donors)

  - Large-sized; needs large patient

    - Some women and children ineligible

  - Similar principle to 1st Gen VADs

    - Pulsatile pump (with valves) connected to huge air compressor

    - Same disadvantages as 1st Gen VADS, ie. size or lack mobility

  - 2nd and 3rd Gen VADs

    - Whole heart taken out and replaced with turbines

    - No heartbeat → continue flow

    - Worked on animals

    - Worked on first impromptu patient → died soon after