# Lecture 8 – Monitoring human movement using wearable sensors

PART 1: Physics behind sensors and functionality

NEWTONS SECOND LAW:

- Accelerometer

- F = ma → How we measure acceleration in systems

- Hooke’s Law : F = -kx

  - Amount of force required to stretch a spring, proportional to the distance needed to be stretched

  - If we can measure displacement of the spring, we can figure out the force

  - Spring = beam

    - Force on spring → spring bends → amount of force depends on 2nd law, F = ma (= -kx) → if mass is known, acceleration can be found from displacement (from the bending)

  - Displacement is determined via the different capacitors

    - Different distances from middle plate to outside plate, as well as, relative charges (ie. intensity of attraction)

GYROSCOPES

- Used to determine direction/way of rotation/angular velocity

  - Senses rotation motion + changes in orientation

  - Via change of angle

  - Consists of a wheel or disc that spins rapidly about an axis, with freedom of movement (can change direction)

  - Since orientation of axis is not affected by tilting the mount, it can provide stability or maintain a reference direction in navigation systems, automatic pilots and stabilizers

- Coriolis Effect

  - Pseudo Force effect  (force acting on object in rotating frame of reference)

  - Inertial: Acts on objects in motion relative to rotating reference frame

  - Rest Frame: in room

    - Spinning disk in room

    - Put hand over disk and fire a bullet

    - Rotation of disk doesn’t affect bullet’s trajectory; bullet travels straight after being fired

  - Rotational frame: Sitting on disk

    - If hand is put up and a bullet is fired, to me; bullet is going straight up

    - From rest POV, bullet is curving away

MEMS GYROSCOPE

- A differential capacitor (same as accelerometer) to measure displacement

- Mass in capacitor plates shake/vibrate left and right (using coulombs laws) → aka. Vibrating gyroscopes

- But once rotated in any way, coriolis effect takes effect

- When mass moves outward, it accelerates to keep up with the frame. When mass moves to centre,  little acceleration needed to keep up with rotation

  - Stationary: mass moves left and right

  - Rotating: mass is deflected up and down

- Can measure displacement for gyroscope via differential capacitor

- Utilizes 2x mass system to keep phone balanced (ie. not vibrate). During movement, mass move perfectly out of phase

MEMS MAGNETOMETER – LORENTZ FORCES

- Easily lost from adding rotations from gyroscopes → magnetometer tells us where we fact

- Lorentz force – if a charged particle is moved with some velocity in a magnetic field, it will feel a force

  - F = V x B (cross product)

  - Where F is perpendicular (RHPR)

- Application of lorentz force is the hall effect

  - Set up a voltage source, that sends current through a circuit with a  s Si semi-conductor (higher resistance than wire, but low enough for current to flow; if current is too high, circuit would explode)

  - Charge moves with velocity → experiences Lorentz forces → charge piles up/pushed to one side of conductor → charge distribution has one side more negative than the other

  - = voltage, indicates presence of magnetic field

  - Needs one in each direction (x,y,z) to detect B-field

ISSUES WITH MEMS MAGNETOMETERS

- Calibration problems → too many magnetic distortions

- Soft Iron = interaction of external field with board

  - Ferromagnetic material on sensor board → material distorts field lines

- Hard iron = fixed magnetic field on board

  - Magnets on sensor board

  - Permanent distortion = always there, always measuring its magnetic field

- Magnetic Declination

  - Earth’s magnetic field is distorted around Earth → compass does not point exactly magnetic north

  - Declination = angle variation on horizontal plane from actual north m

- Magnetic inclination

  - Angle made HORIZONTALLY by Earth’s magnetic field lines

  - At equator, magnetic field goes straight across surface → reliable indication of North

  - South pole = almost 90o

MEMS BAROMETER

- Chamber → sealed air inside → implant piece of resistor on diaphragm (thin skin across hull) → resistor changes resistance when stretched

  - As air pressure, outside of chamber, changes, diaphragm:

    - Stretches when moving up, contracts when moving down

    - Measures altitude (but: needs to be calibrated)

    - CONs: pressure changes according to weather (interference)

GPS (Global Positioning System)

- Multilateration; algorithms of distances to determine position

- Needs 4 satellites

  - All satellites contain atomic clocks, synched up (accounting for time delay) → sends signal with time stamp → phone determines distance between satellites from differences in time stamps

- Special relativity slows time for travelling satellites → time delay subtracted

PART 2: Maths of rotation

- Needs to figure out angles between vectors and how to rotate axis angles to determine orientation, given with the data in the gyroscope

- Dot (scalar) product

  - Determines angle between vectors

- Cross (vector) product

  - Finds vector perpendicular to two vectors

- Angular Velocity

  - Angular distance = angle in radians

  - Angle = distance along arc radius

  - Angular velocity = angle around axis of rotationsecond

    - Speed is perpendicular to  axis of rotationradius       [how fast, it is spinning along the arc]

QUATERNIONS

Unit vector can show direction

- Encodes 3D rotation in the word (especially the u-quaternion)

- AXIS – ANGLE ROTATIONS  = take a point and rotate it around a given vector in 3D space

  - U = rotation axis

    - Needs unit vector form (divide each component by vector’s total absolute length)

  - v and v’ are endpoints of rotation

  - Takes the vector with a ‘0’ real part at front

  - Rotation is encoded in q

    - Need to find q and its quaternion conjugate

    - Put it at the start and end = rotation

    - Utilizes the second equation

- Using RH Grip Rule

  - Thumb points in direction of positive z-axis (or axis of rotation)

  - Fingers curl in +θ

KALMAN FILTER

= random guess of variables from noisy data (eg. orientations, some angles, progression of data as time progresses)

- Figure out orientation → subtract gravity = just acceleration due to movement

  - Integrate it twice to determine position (although inaccurate, due to unknown constants)

DEAD RECKONING

- Sensor on foot → every time, foot hits ground = 0 velocity → get a trace of vertical vs forward position (x metres)